DRAFT UNECE SAFETY GUIDELINES AND GOOD PRACTICES FOR TAILING MANAGEMENT FACILITIES

Draft version 11 February 2008

Definition:

In this guideline Tailings Management Facility (TMF) is intended to encompass the whole set of structures required for the handling of tailings. Starting at the point where the tailings leave the plant to the point for final settling, including the Tailings Storage Facility (TSF), Tailings Dam, Tailings impoundment, decants, spillways, clarification ponds, delivery pipelines etc.

Where, in particular:

Tailings are the fine-grained waste material remaining after the metals and minerals recoverable with the technical processes applied have been extracted. The material is rejected at the “tail end” of the process with a particle size normally ranging from 10 μm to 1.0 mm.

a TSF is a facility used to contain tailings. This can include a tailings dam (impoundment and pond), decant structures and spillways. A TSF can also constitute open pits, dry stacking, lakes or underground storages.

A Tailings Dam refers is a tailings embankment or a Tailings Disposal Dam. The term tailings Dam encompasses embankments, dam walls or other impounding structures, designed to enable the tailings to settle and to retain tailings and process water that are constructed in a controlled manner.

A tailings impoundment is the storage space/volume created by the tailings dam/dams where tailings are deposited and stored. The extent of the impoundment is bounded by the tailings dams and/or natural boundaries.

These definitions are largely following ICOLD Bulletin XXX, (Year). Other definitions relevant to this guideline are provided in Annex X.

CONSIDERATION SHOULD ALSO BE GIVEN TO HOW THE INCLUSION OF SPOIL DUMPS, WASTE ROCK STORAGE FACILITIES, TOPSOIL STOCKPILES AND THE LIKE IS MANAGED. THE ORIGINAL TEXT IS SOMEWHAT UNCLEAR AND MIXES UP WASTE ROCK, TOPSOIL AND OVERBURDEN (WASTE ROCK & SOIL?).

## Introduction

- There is growing body of evidence and understanding that environmental degradation of transboundary watercourses and/or international lakes and unintended large scale transboundary movement of hazardous materials has occurred on numerous occasions throughout the world as a result of TMF failures.

- Such failures from TMFs have contributed to transboundary pollution via mass movement of wastes (generally tailings containing heavy metals and hazardous and/or toxic compounds) as suspended solids and dissolved materials.

- Pollution of such waterways and related damage to, or risk to, human health, infrastructural and environmental resources has the potential to negatively affect relations between neighbouring jurisdictions.

- Moreover, there is growing awareness that many TMFs have the potential to release pollution across boundaries (pose transboundary risk) should they fail or unintentionally release materials. Pathways for cross-border transmission of pollution from TMFs are almost exclusively via transboundary watercourses and/or international lakes.

- Such risks are posed by TMFs in all categories: active, idle/inactive, neglected, temporarily closed; and abandoned/orphaned. There is particular concern regarding the large number of neglected, abandoned or orphaned TMFs where active monitoring or maintenance is not undertaken.

- A TMF represents a large capital investment and an integral part of mining and mineral processing activities. Its proper operation is a key factor in the overall successful operation of a mining project and its industrial processes.

- The overall importance of TMFs to both the economic viability and the social and environmental acceptability of any mining organisation are often underestimated. Perhaps due to a view that there is no direct financial return from the cost of design, construction, operation and rehabilitation the tailings storage facility there is apparently temptation to assign insufficient managerial and financial resources to the design, operation, management and/or closure of tailings dams.

- Assigning low priority to the safety of TMFs has been shown to be a seriously flawed approach as neglect of tailings dams has often been shown to be a major or significant contributing factor to the poor international record of tailings dam failures and incidents.

- As accidents such as Baia Mare (January 2000) have shown, failures and incidents at TMFs can have far-reaching consequences to the environment and environmental services, to human health and to the social acceptance of mining activities.

- Such failures and incidents can lead to much higher costs for a company for items such as emergency response, clean up and repairs, damages claims, law suits, unscheduled closure works, and the loss of Company share value. As such, accident costs almost universally exceed costs that would be entailed for a company to have ensured proper and adequate levels of safety and control in order to prevent the incident.

- The failure of a tailings dam in any part of the world now has the potential to rapidly impact the social acceptance and regulatory frameworks for all other operations of a company concerned and also for the mining industry in general. Industry reputation is an important prerequisite for the implementation (and acceptance) of mining within national development strategies. Negative impacts of such industrial accidents can be exacerbated severely when transboundary effects are involved.

- Such damage to industry reputation and national development strategies is a recurring theme in international circles and is exemplified by the increased awareness being raised throughout the world by UNEP, ICMM, the MMSD Project, the World Mines Ministries Forum (WMMF), the World Wildlife Fund (WWF) and the World Summit on Sustainable Development, 2002. Such organisations are focusing the Mining and Industrial sectors towards safe, sustainable development, which places high priority on increased safety for tailings dams.

- Such organizations also recognize the economic importance of the extractive industries to both National and Local economies, including economies at all stages of development. Moreover, it is recognized that mining may constitutes a first significant foreign direct investment in some host countries and that in such instances the industry can serve an central role as a foundation for economic and social development.

- The operation phase for a TMF can be many decades. Both mining and mineral’s processing are under constant evolution. Practices to design, operate and maintain – as well as to regulate TMFs will also likely require evolution during the life of a TMF.

- Experience regarding the long term behaviour and stability of TSFs after closure is still limited. In the context of tailings dams, long term is defined as 1000 years, or more. While knowledge is constantly increasing, the closed and remediated tailings dams at the current time (2007) are less than one or two decades old. As such, practice can be expected to continually evolve.

- The potential for both chronic pollution and acute risk associated with mine tailings deposits can be very long term. A significant number of examples exist where the remains of tailings and waste from mining operations conducted several centuries or even millennia ago still produce pollution in amounts that is harmful for the environment. This emphasises the importance of correct operation and closure of today’s tailings dams and waste dumps if unacceptable risks or negative impacts are to be caused in the future.

- Abandoned and orphaned sites are jurisdictional gap

- In recognition of the above, a substantial body of work has been performed by the global mining industry, by geotechnical and related geotechnical sciences, by international dam safety organisations, by intergovernmental agencies, and more – to generate guidelines for the building and operation of safe TMFs and their sub-components – particularly tailings dams.

- In recognition of all of the above, and in the context of the risk of accidents contributing to transboundary pollution affecting watercourses and international lakes, and in order to assist the national authorities and the operators in ensuring adequate safety level at TMFs, and an acceptable level of risk posed by such facilities, the UNECE member countries decided to draw up safety guidelines and good practices for tailings dams. These take the form of a set of recommendations and set of requirements that will contribute to the achievement of a basic level of safety for tailings dams.

- The Conference of the Parties to the Convention on the Transboundary Effects of Industrial Accidents and the Meeting of the Parties to the Convention on the Protection and Use of Transboundary Watercourses and International Lakes adopted at their respective meetings (Rome, 15–17 November 2006, and Bonn, 20–22 November 2006) a workplan for the Joint Expert Group on Water and Industrial Accidents. In this they mandated, among other things, that the Group to draw up safety guidelines and a summary of good practice for TMFs – and in particular their sub-component – tailings dams.

- The Joint Expert Group through the established steering group with recognised expertise on tailings dams and transboundary accidents drew up the guidelines. It took into account input provided by the authorities, operators of TMFs, financing institutions and non-governmental organizations during the workshop on safety of tailings dams, held in Yerevan from 12-14 November 2007.

- The established steering group has based the guidelines directly upon the body of work produced by the global community of dam safety scientists, regulating bodies and inter-governmental agencies. Notable among these is work by the International Commission on Large Dams (ICOLD), UNEP, ICMM, etc.

- The following are the recommendations and the key elements of the UNECE guidelines and good practices for TMFs. They are designed to prevent incidents at TMFs, with a key focus on tailings dams, and to limit the potential for negative impacts upon environment, human health and infrastructure. They are based extensively on accepted and published best practice procedures to ensure conformity with international standards. Security concerns are not within the scope of these guidelines and they should be taken into account at all stages of the life-cycle of the TMFs.

    - PRINCIPLES FOR TAILING MANAGEMENT FACILITIES SAFETY

- Governments should provide leadership and create minimum administrative frameworks to facilitate the development and safe operation and decommissioning of the TMF.

- The operators of TMF have primary responsibility for ensuring safety of TMF and formulate and apply safety management procedures, as well as utilize technology and management systems to improve safety and reduce risks.

- TMF should be planned, constructed, operated and closed applying “case by case” or “site by site” approach, as a result of a different geological and hydro geological conditions.

- Only competent – properly certified (if required in accordance with the national legislative, regulatory and safety management norms) – personnel  should be engaged in the design, construction, operation/management, closure of TMF and the relevant competences should be described in the operating and management plan.

- Systems approach to managing TMF safety should be acknowledged by all stakeholders and the high-quality life-cycle “planning – construction – operation – closure” should be ensured.

- Understanding of processes in the life-cycle of a TMF should be developed at the design stage of TMF and it should be further refined through practice and simulations.

- The safety of TMF depends especially on persons responsible for TMFs planning and design (and approval), operators, government inspectors, rescue services and professionals in closure and rehabilitation, they should be therefore adequately trained.

- TMF should be operated in accordance with the construction, sanitation and environmental norms of the country concerned and on the basis of an operating and management plan (operation manual) evaluated and accepted by the relevant competent authority, as appropriate.

- TMF should be classified depending on the risk-hazard assessment taking into account the parameters as specified in the annex.

- Land-use planning considerations should be taken into account in view of TMF placing and indented post-operational use.

- For TMF which pose a potential risk to neighbouring communities and land-uses due to their size or presence of hazardous materials, information to and involvement of these communities and individuals should be ensured for the purpose of drawing up an emergency plan that the community understands.

- TMFs should be operated in accordance with the provisions of the UNECE Convention on Access to Information, Public Participation in Decision-making and Access to Justice in Environmental Matters (the Aarhus Convention). Where subject of concern is of transboundary nature, the principles of Almaty Guidelines on Promoting the Application of the Principles of the Aarhus Convention in International Forums should apply (http://www.unece.org/env/pp/ppif.htm).

- Principle to be added on subterranean deposits

    - RECOMMENDATIONS

- These guidelines constitute a minimum set of requirements to ensure a basic level of safety for TMFs. They highlight all the aspects to be considered to achieve acceptable level of safety through applying different policies, measures and methodologies.

- These guidelines should be read also in the context of relevant national requirements and existing international guidelines, recommendations and standards concerning TMFs, and using internationally assessable information sources.

- Below are recommendations to the UNECE member countries, competent authorities and TMFs operators. The technical and organizational aspects, listed in the annex, are an integral part of these guidelines and good practices.

- Recommendations to UNECE member countries

- UNECE member countries should identify competent authorities at the national, sub-national and local level that are given access to the necessary competences for the tasks foreseen in these recommendations.

- UNECE member countries should adopt adequate legislation for ensuring safe construction, operation and closure of TMFs, including legislation for handling the abandoned and orphaned sites

- UNECE member countries should initiate a national inventory of closed, abandoned or orphaned TMFs that may constitute a risk to human health or the environment. The inventories should be a prerequisite for the implementation of recommendations to competent authorities on closed, abandoned and orphaned sites.

- National inventories of closed, abandoned or orphaned TMFs should consider both current impacts and risks for future acute (accidents and spills) or long term (leaching) negative effects.

- UNECE member countries should share new experience and newly developed good practice for TMF safety in all the phases of its life-cycle

  - Recommendations to competent authorities

- Competent authorities should evaluate and accept the operating and management plan (operation manual) drawn up by the operator

- Competent authorities should verify the TMF monitoring performed by the operator if it fulfils set quality standards

- Competent authorities should ensure that TMF operators develop internal emergency plan and provide necessary information and cooperate on preparing external plans

- Competent authorities shall develop external emergency plans in association with operators, community groups, local authorities and rescue services and apply them off-site TMF in case of major accidents

- Competent authorities should ensure that the internal and external emergency plans are reviewed, and tested periodically and where necessary, revised and updated

- Competent authorities should apply methodologies for risk assessment and identification at the closed, abandoned or orphaned TMF using a step by step approach, starting with a basic screening of sites, whereby resources are gradually directed towards sites with the highest risks.

- Based on the risks identified, competent authorities should make plans for risk reduction measures and/or monitoring (early warning) for the closed, abandoned or orphaned TMF

- Competent authorities should train the inspectors on the ongoing basis so that the inspections are preformed effectively.

- The non-mining professionals (as defined in the glossary)dealing with Environmental Impact Assessment and land-use planning for mining projects should be trained on the tailings issues

- Competent authorities should encourage and engage in a “train the trainers” programme the existing educational institutions so that they reach the necessary capacities for training the company and government staff. Where possible, use can be made of international training programmes offered by various national and UN institutions.

- All authorities involved into TMF safety should cooperate with each other preferably within an integrated system, in which one authority is the coordinating body.

- Competent authorities should notify their counterparts in neighbouring countries about the TMFs which in case of accident can cause transboundary effects.

- Competent authorities should ensure public participation and information in accordance with the relevant provisions of Industrial Accidents, Water and in particular the Aarhus Convention.

- Recommendations to Tailing Management Facilities operators

- All TMF should have an operating and management plan (operating manual) that is available to all personnel, local inhabitants, government inspectors and other relevant stakeholders. All documents relating to planning, design and construction should be maintained in an accessible way, with records kept permanently for reference at the future time.

- TMF operator should monitor the TMF in accordance with the operating and management plan as accepted by the competent authorities.

- TMF operators should draw up and implement internal emergency plans and apply them inside TMF whenever a tangible risk for major accidents to occur has been identified or an  uncontrolled event occurs that could lead to a major accident or a major accident has occurred. TMF operators should review, test, revise and update the internal emergency plans at the point of time when there was a change in the mine operation and management.

- The TMF operator should notify competent authorities in case of emergencies in accordance with the operating and management plan.

- TMF operators should cooperate with competent authorities and local communities in preparing the external emergency plans.

- TMF operators should train their personnel, reinforce and revise personnel’s knowledge on safety and in particular on how to identify potentially harmful events.

- TMF operators should implement environmental audits for their facilities and promote use of environmental management systems.

Annex

Technical and Organizational Aspects

- PRE-CONSTRUCTION AND CONSTRUCTION

Please integrate into your parts the recommendations contained in the recommendation document

Licensing

Special authorization and or licensing procedure should be introduced by the competent authorities to permit construction of TMF.

There should be a system of licenses dependent on the hazard-risk evaluation for TMF. This evaluation should be done by competent authorities on the basis of the evaluation of the operating and management plan (operating manual) drawn up by the operator.

The licensing procedure should differentiate between:

  - basic authorization procedure

  - authorizations involving public participation

  - authorizations involving environmental impact assessment and public participation

The complex procedure should be applied for TMFs where:

- the waste facility contains waste considered hazardous (recommended classification and threshold quantities under Directive 91/689/EEC), or

- the waste facility contains substances and preparations classified as dangerous (recommended classification and threshold quantities Directive 67/548/EEC or 1999/45/EC), or

- a failure or incorrect operation can give rise to a major accident

Environmental Impact Assessment and Land Use Planning

An environmental impact assessment (EIA) should be considered as a precondition for construction and operation of TMF. The EIA should address the potential physical impact of the TMF on the physical and social environment and should be open for general public and interested or affected persons to comment and provide input on the assessment and if there are prerequisites to object it.

The EIA should address:

Location criteria: climate, general topography, regional geology, seismic hazard, environmental sensitivity, hydrology (ground and surface waters), local geomorphology

Tailing criteria: geochemical character of TMF, physical and geotechnical character of TMF

Site criteria: downstream infrastructure, cadastral boundaries, potential underlying mineralization, site topography, hydrogeology

Management: tailing deposition method, water balance, method for managing storm events, monitoring;

Closure: completion criteria, intended post-operational land use, , long term physical, geotechnical and biological stability

Hazard/Risk Assessment

Sources of hazards at the TMF can be:

  - direct: location, tailing and site criteria (detailed specification of the criteria is mentioned under EIA section),

  - indirect: management

For each TMF the sources of hazard should be evaluated including the assessment of what can be the consequence and with which probability it can happen (risk assessment).

Based on the risk assessment three classes of risk can be distinguished:

- green – low consequence and probability.

- yellow – medium consequence and probability

- red – high consequences and probability

Depending on the risk classes the construction of the TMF can be permitted (green class), permitted with additional conditions (measures taken to reduce the consequence or the probability to move from the yellow to green class) or should be prohibited (red class) unless economically acceptable measures can be taken to reduce the consequences or the probability.

The hazard and risk assessment, especially taking into account the direct sources of hazard, should be a starting point for planning and designing a safe TMF.

The hazard and risk assessment should be constantly reviewed throughout all the phases of the TMF life-cycle.

Dam Safety

While planning and designing a safe TMF a particular attention should be directed to:

- tailing pond - following parameters need to be assessed accurately:

- the stability of sludge (slurry density)

- the level of the ground water

- geological situation

- hydrogeological situation

- hydrological situation

- geophysical situation

- tailing dam -  following parameters need to be assessed accurately:

    - the slope stability of the dam

- the stability of the tailing material (induced liquification)

- erosion to the dam (suffusion and outside erosion

- slope sliding

The dam raising method should be chosen with regard to the local conditions (seismicity, tailings composition, severe climate)

- Additional impoundments should be designed to receive inflow from emergency outlets

- Hazardous substances and process water should be reused as far as technically possible (recycling) and in case it is not possible to recycle hazardous substances, they should be neutralized before they are discharged into TMF

- OPERATION AND MANAGEMENT

Management

The TMF should be operated and managed on the basis of an operating and management plan (operation manual) and waste management plan (if such is not part of operation manual) evaluated and approved by the competent authorities, which is developed in the planning phase and progressively modified. Its aim is to effectively manage the risks and hazards at the TMF (or waste facility for waste management plan) in order to stay within the risk green class (risk classification under Hazard and Risk assessment section)

The operation manual should contain:

- description of all monitoring procedures –mechanisms for investigations: sampling locations, sampling frequency, checklists and compliance parameters such as: minimum capacity/freeboard, pore pressure, groundwater level, functioning of the drainage system, surface water diversion, dam movement, slope stability;

- procedures for reporting on non-compliance and failures

- corrective actions to be applied in case of non-compliance

- internal emergency plan

- assessment parameters for effectiveness and suitability of the operation manual

Any changes to operation manual should be subject to its performance analysis, which should be documented.

The performance of the TMF should be assessed and described during significant seasonal events and the data gathered should be used for rehabilitation planning.

In case of TMF acid rock drainage potential the management plans should be developed to prevent, reduce and treat the acid water generation.

Education and training of personnel

The life-cycle approach to TMF requires that personnel in a variety of different professions and institutions have a common understanding and knowledge of the technical and managerial aspects, and use complementary professional procedures in their work. This requires a certain level of training (and re-training) of various persons associated with TMF.

The personnel concerned should be identified along the “planning-design-construction-operation-decommisioning-rehabilitation” chain

A variety of different professions is involved, including engineers and managers, planners, regulators, environmental and safety specialists, monitoring and auditing staff.

Personnel working at and responsible for safety of TMF should be educated and trained in:

- technology

- on-site procedures

- standards and regulations

- managements systems and tools

- measurements

- auditing

- reporting

- communication

The technical and managerial skills, communication and negotiation skills are needed to ensure that a large number of stakeholders (especially for large facilities in populated areas) are properly informed and involved in decision relevant to their interests. These interests often concern social, environmental and economic issues which must be understood by managers and designers in particular.

The inherent uncertainties surrounding all potentially hazardous TMFs require special skills in risk assessment and management but also in risk communication and reporting.

Training of relevant personnel should include apart from the technical aspects of TMF also the “context” subjects that concern related disciplines such as environment, social and financial areas.

The personnel should participate in formal courses and in-service training, during which the knowledge obtained in mining and engineering schools, management schools and from technical materials can be further enhanced. Where possible, training should focus on active, hand on methodologies as the most effective method of training adults.

Management of abandoned sites

The management system for closed, abandoned or orphaned sites should include the organizational structure, responsibilities, procedures and resources for determining and implementing the accident prevention policy:

Organizational structure: competent authorities should be nominated to carry out the assessment, monitoring of closed, abandoned or orphaned sites. They should be allocated adequate staff, technical means and budget to accomplish its mandate.

Identification and evaluation of hazards: closed, abandoned or orphaned sites should be known and catalogued in a inventory containing their location and key characteristic and they should be labelled accordingly.

Monitoring and maintenance: the sites should be monitored and maintained and remediation should be undertaken in the first place at these sites, where failures are likely or very likely to happen.

Removing or reengineering of sites due to very high costs should be only undertaken where risk capping fails.

- INSPECTIONS

Facility inspections,

Facility inspection should be performed by competent authorities at all phases of the life cycle of the TMF and should ensure that TMFs operators are taking all the necessary steps to manage the TMFs safety without posing any risk to the environment of human health. The inspectors should verify in particular if the TMF is managed in accordance with the legal and regulatory standards as well as the approved operation manual and waste management plan:

- in pre-construction and construction phase: verification of the location for the waste facility, construction of the tailing dam

- operation phase: verification if physical stability of the waste facility is ensured and that pollution or contamination of soil, air, surface water or groundwater are prevented, verification on regular monitoring on immission and emission measurements, verification if failures were properly reported and proper corrective action were taken

- clousure, after clousure: verification if physical stability of the waste facility is ensured, verificatin on rehabilitation process including its proper documentation

In case there is a violation in management of TMF to the operation manual and or waste management plan the inspection authority should urge the operator to introduce corrective actions within a specified period and if this is not done to take back the operartion permit.

Education and training for inspectors

The inspectors should be trained on:

- new technologies

- standards and procedures

- monitoring and auditing standards

- risk assessment

- communication with operator personnel

The training resources should be evaluated and augmented as necessary to provide the complete range of subjects and skills required for life-cycle TMF inspection

The training should include: simulations, practice, drill, role play, field exercise and discussing case studies of lessons learned and it should be an ongoing process and not a one-shot affair. Extensive material is available from UN to support such events and programmes.

The training should be spread over time and be subject to reinforcement, revision and follow-up.

Assessment and priority tasks on abandoned sites

The closed, abandoned or orphaned sites should be inspected depending on the risk posed by the site assessed in the initial screening.

The initial screening should include a walkover survey concentrating on TMF features such as containment dam, beach, water management system and hydrographic catchment area.

The components, structures and parameters specifically inspected and screened in case of any signs of unusual behavior should be the following:

- geomorphological situation and catchment area (inflowing streams, size and topography of TMF catchment area, expected frequency and magnitude of flood events)

- dam crest (materials used, irregularities, depressions, signs of erosion)

- slope geometry (height, angle, berms)

- containment dam slope condition (materials, vegetation, signs of erosion, seepages, slumping, active mass movements such as slumping, sliding or rotational failures)

- lagoon condition (size relative to TMF, depth, geometry, vegetation, alien deposits such as litter)

- water management system (existence and condition of drainages, bypasses, decantation plant, contingency/emergency spillways, pumps)

- monitoring equipment (inter alia trigonometric points, survey targets, piezometers / standpipes)

- historic incidents and accidents.

Depending on the result of the screening the sites should be labeled from green to red classes of risk

Sites labelled with yellow but  mainly with red classes of risk should undergo a detailed assessment process in order to separate those sites where monitoring of hazard factors would be sufficient as risk mitigation approach from those where failures are likely or very likely to happen in short term. The detailed assessment should include:

- research on existing documentation,

- detailed site surveys

- potential spot investigations

- calculations “back of the envelope” on probabilities of identified specific failure modes

The high risk sites should undergo appropriate investigation and data generation measures (e.g. topographical surveys). For each of them specific remediation programmes should be designed, tendered and contracted within the reasonable timeframe.

- EMERGENCY PLANNING

Emergency plans should be established for each TMF for phases of construction, operation and closure. The appropriate emergency plan needs to be established prior to accepting construction, operation or closure by authorities. Hence, they shall be drawn up within the periods set by local or international rules

Emergency plans should be established, tested and revised by TMF operator (internal plans) and by authorities (external plans), in particular when:

- accident or emergency situation occurred at the site or other similar sites;

- the emergency service organization was changed;

- new technical knowledge was developed

- new risks are identified that are associated with the TMF

- design values are approached or exceeded as a result of changes, mismanagement, structural problems, equipment modification or natural events.

The plan should evaluate downstream inundation hazard resulting from floods or dam failure, and upstream conditions that might result from major land displacements or increased flood flows. If applicable, the plan should include inundation maps for the flows resulting from design floods and from possible failure of the dam.

Where there is a series of dams on the stream, analyses should be made considering the potential for progressive "domino" failure of the dams. To evaluate the effects of dam failure, maps should be prepared delineating the area which would be inundated in the event of failure. Analyses should be made to determine conditions which could be expected to result in slow, rapid or practically instantaneous dam failure.

The emergency plans should include:

  - The scope and objective of the emergency plan

  - Evaluation of emergency scenarios, risks, potentially affected areas etc

  - Responsibilities of each member of the organization (chain of responsibility and authority for actions to be taken)

  - Organization of communication and notification procedures

  - Available equipment for interventions

  - Procedures for emergency response for each of the determined emergency scenarios

  - Procedures for remediation

Internal emergency planning

Prior to development of an internal emergency plan analysis should be made to determine the most likely mode of dam failure under the most adverse condition and the resulting peak water outflow following the failure.

The internal emergency plans should be part of the operating and management plan (operating manual)

The internal emergency plan, specific for each site and situation, must be developed and continuously revised.

The internal emergency plans should contain estimation on amounts and types of construction materials and equipment needed for emergency repairs based on the structural, foundation, and other characteristics of the dam; design and construction history; and history of prior problems.

Plans for notification of key personnel and the public must be an integral part of the emergency plan and should be prepared for slowly developing, rapidly developing, and instantaneous dam failure conditions.

External emergency planning

External emergency plans should be prepared and implemented by the local authorities, conform to local needs and vary in complexity in accordance with the type and degree of occupancy of the potentially affected area.

Public should be given the opportunity to participate in the preparation and revision of the external emergency plans. It should be entitled to express comments within reasonable time frames and due account should be taken of these comments.

It should be ensured that in border areas the contingency plans of two regions of neighbouring countries are compatible with each other and include contact details to allow proper notification.

Public of neighbouring countries should be given the same rights as public of the concerned country to participate in preparation and revision of external emergency plans.

Emergency planning for abandoned sites

The competent authorities should develop emergency plans for abandoned sites. Before developing the plans it should be verified which capabilities and responsibilities already exist

- existing regional emergency response frameworks, such as civil protection, fire brigades, flood response

- emergency response plans in place for a coordinated regional/transboundary response

- clear command structure and management of interfaces between concerned authorities

- existing models of scenarios for mining accidents and their integration into the emergency plans

Glossary

Terms and Definitions applied in this Guideline

It is intended that these clarifications will assist the reader in gaining a full understanding and appreciation of the Guideline.

Active Facility

A facility site where mining and mineral processing operations are ongoing (in this case the disposal of waste materials pursuant to mining or processing) that has relevant and proper regulatory approvals in place.

Idle or Inactive Facility

All sites where operations such as the disposal of wastes pursuant to mining or processing has ceased. Thus all mine TMFs not considered active.

Neglected Facility

An idle or inactive facility that has not been closed and has no clear and obvious owner but that may still be held under some form of title and where all current appropriate regulatory obligations have not been satisfied. This definition can include sites where regulation changes have led to closure parameters being imposed after the site became inactive.

Temporarily closed facility

An Idle/Inactive mine facility under Care and Maintenance – the phase following temporary cessation of operations when infrastructure remains intact and the site continues to be managed. The site is still held under some form of title and all current appropriate regulatory obligations for closure have not been satisfied. When being maintained in some way with a view to future resumption of operations, such sites are frequently referred to as being under care and maintenance.

Abandoned facility

A facility formerly used for mining and mineral processing operations (an idle/inactive facility) that is neglected and whose legal owners still exist and can be located.

Acid mine drainage (AMD) or Acid rock drainage (ARD)

Acidic drainage/seepage stemming from open pit, underground mining operations, waste rock or tailings facilities that contains sulphuric acid and dissolved metals resulting from the oxidation of contained sulphide minerals.

After-care

The last phase of closure is required to verify that the measures taken are performing according to design and expectations. When verified, the long term monitoring phase starts.

Best available techniques

is as defined in Article 2(11) of Directive 96/61/EC;

Clarification pond, Return water dam or Clearwater dam

An impoundment separate from the tailings impoundment, used for a second clarification step and/or an extended storage of process water.

Closure

The shutting down and decommissioning of a mine and tailings dams when production has ceased including the transition of the mining area and dam into long-term stable structures. Normally closure includes decommissioning, remediation (reclamation or rehabilitation) and after care at the site and the tailings dam.

INSERT A FIGURE AS ABOVE COVERING THE DEFINITIONS FOR TAILINGS DAMS …DECOMMISSIONED, CLOSED, AFTER-CARE ETC.

Competent authority

the authority or authorities which a Member State designates as responsible for performing the duties arising from this Directive;

Competent person

a natural person who has the technical knowledge and experience, as defined by the national law of the Member State in which the person operates, to perform the duties arising from this Directive;

Consequence (refer to Hazard)

The outcome or impact of an event. Consequences affect human, environmental, economical or cultural aspects of a community.

Dangerous substance

means a substance, mixture or preparation which is dangerous within the meaning of Directive 67/548/EEC  or Directive 1999/45/EC ;

Dam

means an engineered structure designed to retain or confine water and/or waste within a pond;

Dam failure

An event resulting in the escape of tailings tailings and/or water from the tailings dam.

Dam incident

An unexpected event that occurs to a tailings dam that poses a threat to dam safety or the environment and requires rapid response to avoid a likely dam failure.

Decommission

Closing down of operations and removal of unwanted structures, such as infrastructure, buildings, pipelines services etc.

Environment

Interrelated physical, chemical, biological, social, spiritual and cultural components that affect the growth and development of living organisms.

Erosion

Detachment and subsequent removal of soils or tailings by wind, rain, wave action, freezing, thawing and other processes.

Extractive industries

means all establishments and undertakings engaged in surface or underground extraction of mineral resources for commercial purposes, including extraction by drilling boreholes, or treatment of the extracted material;

Freeboard (actual)

The vertical height from the current pond level to the crest of the dam at its lowest point.

Freeboard (design)

The vertical height from the normal operating level of the tailings dam supernatant pool, to the crest of the dam at its lowest elevation on the dam perimeter.

Freeboard (beach)

The vertical height from the top of the tailings on a beach, to the crest of the dam.

Hazard (refer consequence)

The potential consequence of an accident or failure event, without consideration of the probability of that event occurring.

Hazard Rating

A ranking of the severity of the consequences (or hazards) arising from a potential accident or failure.

Hazardous waste

is as defined in Article 1(4) of Council Directive 91/689/EEC of 12 December 1991 on hazardous waste

;

Heap

means an engineered facility for the deposit of solid waste on the surface;

Hydraulic gradient

Difference in hydraulic head between two points divided by the travel distance between the points.

Inert waste

The waste that does not undergo any significant physical, chemical or biological transformations. Inert waste will not dissolve, burn or otherwise physically or chemically react, biodegrade or adversely affect other matter with which it comes into contact in a way likely to give rise to environmental pollution or harm human health. The total leachability and pollutant content of the waste and the ecotoxicity of the leachate must be insignificant, and in particular not endanger the quality of surface water and/or groundwater;

Inspection

Examination of the current condition and behaviour of TSF components

Leachate

means any liquid percolating through the deposited waste and emitted from or contained within a waste facility, including polluted drainage, which may adversely affect the environment if not appropriately treated;

Likelihood (refer probability)

The probability or frequency of something occurring

Long term phase

The period of time for which closure is designed and that commences after completion of the after-care phase. Usually requires ongoing monitoring.

Major accident

means an occurrence on site in the course of an operation involving the management of extractive waste in any establishment leading to a serious danger to human health and/or the environment, whether immediately or over time, on-site or off-site;

Monitoring

The action of inspection, recording and analysing the key parameters for tailings disposal operations or following closure.

Mineral resource

means a naturally occurring deposit in the earth's crust of an organic or inorganic substance, such as energy fuels, metal ores, industrial minerals and construction minerals, but excluding water;

Operator

the natural or legal person responsible for the management of extractive waste, in accordance with the national law of the Member State in which waste management takes place, including in respect of temporary storage of extractive waste as well as the operational and the after-closure phases;

Orphaned site

Abandoned mining operations, TMFs, tailings dams etc. for which the responsible party no longer exists or cannot be located.

Phreatic surface

The surface between the zone of full soil saturation and the zone of partial soil aeration; that surface of the unconfined ground water at which the pressure is equal to that of the atmosphere.

Pond

means a natural or engineered facility for disposing of fine-grained waste, normally tailings, along with varying amounts of free water, resulting from the treatment of mineral resources and from the clearing and recycling of process water;

Probable Maximum Flood (PMF)

Stormwater flow resulting from the most severe precipitation and/or snowmelt event considered reasonably possible at a particular geographic location. A site-specific determination, based on the maximae of the possible range in meteorological and hydrological conditions. Variables include the duration, the catchment area and the time of the year.

Prospecting

search for mineral deposits of economic value, including sampling, bulk sampling, drilling and trenching, but excluding any works required for the development of such deposits, and any activities directly associated with an existing extractive operation;

Public

means one or more natural or legal persons and, in accordance with national legislation or practice, their associations, organisations or groups;

Remediation

Measures required to secure the long-term stability and to ensure environmental safety of structures such as tailings dams and disturbed ground. Often includes measures to encapsulate the tailings in order to restrict the potential for toxic materials entering the wider environment.

Rehabilitation

the treatment of the land affected by a waste facility in such a way as to restore the land to a satisfactory state, with particular regard to soil quality, wild life, natural habitats, freshwater systems, landscape and appropriate beneficial uses;

Reclamation

Conversion of severely damaged land into useful land use, not necessarily to its original use. The physical aspects of earth moving, regrading and revegetation.

Risk

The combination or product of the Probability of an event occurring and the consequences of that event.

Seepage point

The point where the hydraulic gradient exits the downstream slope.

Site

all land at a distinct geographic location under the management control of an operator;

Substantial change

a change in the structure or operation of a waste facility that, in the opinion of the competent authority, may have significant negative effects on human health or the environment.

Surveillance

Activities relating to detailed inspections/measurements and records of the TSF.

Tailings

means the waste solids or slurries that remain after the treatment of minerals by separation processes (e.g. crushing, grinding, size-sorting, flotation and other physico-chemical techniques) to remove the valuable minerals from the less valuable rock. The material is rejected at the “tail end” of the process with a particle size normally ranging from 10 μm to 1.0 mm.

Tailings beach

The area of tailings between the edge of the water pond and the edge of the exposed tailings where they meet the slope of the dam.

Tailings dam (Tailings embankment or Tailings Disposal Dam)

An embankment, dam wall or other impounding structure, designed to enable the tailings to settle and to retain tailings and process water, constructed in a controlled manner.

Tailings impoundment

The storage space/volume created by the tailings dam/dams where tailings is deposited and stored. The extent of the impoundment is to the bounds of the tailings dams and/or natural boundaries.

Tailings management facility (TMF)

The whole set of structures required for the handling of tailings. Starting at the point where the tailings leave the plant to the point for final settling, including TSF, decants, spillways, clarification ponds, delivery pipelines etc.

Tailings pond or supernatant pond

The free water contained within the tailings impoundment.

Tailings sand

The sand obtained from the total tailings for use in construction of the tailings dam. Often produced by natural segregation on the beach or by cycloning the total tailings.

Tailings slimes

The fine portion of the total tailings. Slimes generally range from a maximum particle size of about 75 μm to clay size.

Tailings slurry

A fluid suspension comprising liquids (water) and solids (tailings).

Tailings storage facility (TSF)

A facility used to contain tailings including the tailings dam (impoundment and pond), decant structures and spillways. A TSF can also constitute open pits, dry stacking, lakes or underground storages.

Transboundary waters

Any surface or ground waters which mark, cross or are located on boundaries between two or more States

Treatment

means the mechanical, physical, biological, thermal or chemical process or combination of processes carried out on mineral resources, including from the working of quarries, with a view to extracting the mineral, including size change, classification, separation and leaching, and the re-processing of previously discarded waste, but excluding smelting, thermal manufacturing processes (other than the burning of limestone) and metallurgical processes;

Unpolluted soil

The soil that is removed from the upper layer of the ground during extractive activities and that is not deemed to be polluted under the national law of the Member State where the site is located or under Community law;

Waste

is as defined in Article 1(a) of Directive 75/442/EEC

Waste facility

means any area designated for the accumulation or deposit of extractive waste, whether in a solid or liquid state or in solution or suspension,

Waste holder

means the producer of the extractive waste or the natural or legal person who is in possession of it;

Weak acid dissociable cyanide WAD Cyanide

means cyanide and cyanide compounds that are dissociated with a weak acid at a defined pH;

Weathering

Processes by which particles, rocks and minerals, are altered on exposure, including changes due to and atmospheric agents such as air, water and biological activity.

Documents Defining Terms Related to the Subject of this Guideline

Most definitions given for this guideline are influenced strongly by the publications of the ICOLD Committee on Tailings Dams and Waste Lagoons. The definitions are in turn drawn from one or several of the following documents:

Australian and New Zealand Minerals and Energy Council (ANZMEC) and Minerals

Council of Australia (MCA) 2000. Strategic Framework for Mine Closure. National

Library of Australia Catalogue Data. ISR 2000/155, ISBN 0 642 72138 6.

European Commission 2004. Draft Reference Document on Best Available Techniques for management of Tailings and Waste-Rock in Mining Activities (BAT) European IPPC

Bureau July 2004 (http://www.jrc.es/pub/english.cgi/0/733169) (December 2004).

International Commission on Large Dams (ICOLD) 1989. Tailings Dam Safety –

Guidelines Bulletin 74. Published by International Commission on Large Dams.

Imprimerie de Montligeon, 61400 La Chapelle Montligeon. ISSN 0534-8293 (http://www.icold-cigb.net)

International Commission on Large Dams (ICOLD) 1996. A Guide to Tailings Dams and

Impoundments – Design, Construction, Use and Rehabilitation Bulletin 106. Published by International Commission on Large Dams together with UNEP, United Nations Environmental Program. Imprimerie Louis-jean 05003 Gap, France. ISBN 92-807-1590-

7, ISSN 0534-8293. (http://www.icold-cigb.org), (http://www.uneptie.org) (July 2004).

Ministerial Council on Mineral and Petroleum Resources and Minerals Council of Australia 2003. Strategic Framework for Tailings Management. National Capital Printing,

ISR 2003/091, ISBN 0 642 72243 9

International Institute for Environment and Development in the Project Mining, Minerals

and Sustainable Development. (MMSD) 2002. Research on Mine Closure Policy Cochilco, Chilean Copper Commission for IIED, (September 2004)

(http://www.iied.org/mmsd/mmsd_pdfs/044_cochilco.pdf )

Rehbinder G., Gustafsson G. and Thunvik R. 1995. Grundvattenströmningens teori [Groundwater flow theory] Royal Institute of Technology, Stockholm and Chalmers University of Technology, Gothenburg. KTH, Ts- Tryck & Kopiering, Stockholm 1995. (In Swedish)

United Nations Environment Programme (UNEP) and The International Council on

Metals and the Environment (ICME), now International Council on Mining and Metals

(ICMM) 1998. Case studies on Tailings Management. ISBN 1-895720-29-X,

(http://www.unep.org), (http://www.icmm.com) (July 2004)

Vick S.G. 1990. Planning, Design, and Analysis of Tailings Dams. Vancouver, Canada:

BiTech Publishers Ltd.. ISBN 0-921095-12-0.

Final report by Whitehorse Mining Initiative (WMI) Environment Issues Group October

1994.

Directive 2006/21/Ec Of The European Parliament And Of The Council Of 15 March 2006 On the management of waste from extractive industries and amending Directive 2004/35/EC, OJ L 102/15 11.4.2006

Council Directive 67/548/EEC of 27 June 1967 on the approximation of laws, regulations and administrative provisions relating to the classification, packaging and labelling of dangerous substances (OJ 196, 16.8.1967, p. 1). Directive as last amended by Commission Directive 2004/73/EC (OJ L 152, 30.4.2004, p. 1).

Directive 1999/45/EC of the European Parliament and of the Council of 31 May 1999 concerning the approximation of the laws, regulations and administrative provisions of the Member States relating to the classification, packaging and labelling of dangerous preparations (OJ L 200, 30.7.1999, p. 1). Directive as last amended by Council Directive 2004/66/EC (OJ L 168, 1.5.2004, p. 35).